Steven Davis

5/17/2026

CS 350 Emerging Sys Arch & Tech 2-4 Milestone One: Pulse Width Modulation Lab

Sina Asadi

1. At what frequency can you see the LED start to blink?

When testing lower PWM frequencies, I began to see the LED blink at around 10 Hz. At 60 Hz, the LED appeared steady because the pulses happened too quickly for my eyes to separate. As the frequency decreased, the blinking became more noticeable, especially below about 10-15 Hz.

2. At what duty cycle is the intensity of the LED perceptibly diminished from the initial 50% duty cycle?

The LED brightness became perceptibly dimmer when the duty cycle was reduced below the initial 50% setting. I noticed a clear reduction of around 30% to 40% in the duty cycle, and the LED became much dimmer at lower values, such as 20% or 10%. This showed that the duty cycle directly affects the LED's apparent brightness.

3. When changing the duty cycle of the PWM, the loop used an increment of 5 every tenth of a second. Was this perceptibly smooth? If not, what could you change to improve the visual response? Why?

The fade was mostly smooth, but slight stepping was still visible because the duty cycle changed by 5% at a time. To improve the visual response, I could use smaller increments, such as 1% or 2%, and possibly shorten the delay between changes. Smaller duty-cycle steps would make each brightness change less noticeable, making the LED fade appear smoother to the eye.

4. What function sets the PWM frequency for a GPIO line?

The PWM frequency is set when the PWM instance is created with the GPIO.PWM() function. In this lab, the code used pwm18 = GPIO.PWM(18, 60), which sets GPIO line 18 to use PWM at 60 Hz. If the program needs to change the frequency later, the PWM object can also use ChangeFrequency().

5. What function sets the duty cycle for a GPIO line?

The duty cycle is set with the ChangeDutyCycle() function on the PWM object. In this lab, the program used pwm18.ChangeDutyCycle(dutyCycle) inside the for loops to fade the LED from dim to bright and then back down again. The starting duty cycle can also be set when PWM begins, such as pwm18.start(0) for 0% duty cycle.

This lab showed how PWM can control an LED more precisely than a simple on/off signal. By changing the frequency, the LED can appear steady or visibly blink. By changing the duty cycle, the LED brightness can be adjusted or faded smoothly. These same PWM concepts can also be applied to embedded systems that control motors, fans, motor controllers, and other hardware devices.