Steven Davis

6/20/2026

CS 350 Emerging Sys Arch & Tech 7-1 Final Project: Thermostat Lab

Sina Asadi

The SysTec thermostat prototype uses an AHT20 sensor through I2C, an LCD, three GPIO buttons, two PWM LEDs, and a UART status output. A production design must continue to support those peripherals, connect to the cloud through Wi-Fi, and provide enough Flash and RAM for the control and networking software. I compared a Raspberry Pi 4 Model B, a Microchip PIC32MZ-W1/WFI32 Wi-Fi MCU, and a Freescale/NXP i.MX RT1060-class MCU. The Raspberry Pi 4 Model B is the simplest option for the current prototype because the application already runs on it. It has onboard dual-band 802.11ac Wi-Fi, a 40-pin GPIO header, and 1 GB to 8 GB of LPDDR4 memory (Raspberry Pi, n.d.). It has more than enough resources for Python, drivers, the LCD, serial communication, and cloud software. The tradeoff is that it is a full Linux computer, so it uses more power and adds unnecessary cost and complexity for a small thermostat product.

The Microchip PIC32MZ-W1/WFI32 is a better fit for production. It combines a 32-bit microcontroller and 2.4 GHz Wi-Fi in one device. The family provides up to 2 MB of Flash, 640 KB of RAM, and up to 60 GPIO pins; it also supports WPA/WPA2/WPA3 and TLS/SSL security (Microchip Technology, n.d.). Those resources can support the buttons, PWM LEDs, I2C temperature sensor, LCD, state machine, UART service interface, Wi-Fi stack, and a cloud client. Because Wi-Fi is integrated, the product requires fewer parts than a separate controller-and-radio design. A Freescale/NXP i.MX RT1060-class MCU is also capable. It offers an Arm Cortex-M7 core up to 600 MHz, 1 MB of on-chip RAM, GPIO, UART, I2C, SPI, USB, and Ethernet interfaces (NXP Semiconductors, n.d.). It could support the thermostat’s local control functions, but it does not include integrated Wi-Fi. SysTec would need an external Wi-Fi module, antenna, drivers, and additional security integration. That is reasonable for a more advanced industrial controller, but it adds hardware and development work for this product.

I recommend the Microchip PIC32MZ-W1/WFI32 architecture. It meets the peripheral, Wi-Fi, and memory requirements while using an embedded platform better suited to a low-power consumer thermostat. The Raspberry Pi remains the best platform for development and testing. The i.MX RT1060 is a strong future option if SysTec later needs more real-time processing or industrial interfaces. Still, the Microchip Wi-Fi MCU offers the best balance of integration, security, memory, and production suitability for the current design.

References

Microchip Technology. (n.d.). PIC32MZ W1 and WFI32 Wi-Fi microcontrollers. https://www.microchip.com/en-us/products/microcontrollers/32-bit-mcus/pic32m/pic32mz-w1

NXP Semiconductors. (n.d.). i.MX RT1060 crossover MCU for industrial edge computing. https://www.nxp.com/products/i.MX-RT1060

Raspberry Pi. (n.d.). Raspberry Pi 4 Model B specifications. https://www.raspberrypi.com/products/raspberry-pi-4-model-b/specifications/